| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | 18 / 07 / 2023 | Batch No: | C-1 |
| Faculty Name: |  | Roll No: | 16010122257 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 1

Title: Study of Basic Gates and Universal Gates

Aim and Objective of the Experiment:

Understand Basic Logic Gates and Universal Gates

COs to be achieved:

CO1: Recall basic gates & logic families and binary, octal & hexadecimal calculations and conversions.

Tools used:

Trainer kits

Theory:

Logic gates are electronic circuits that perform logical operations on one or more input signals to produce an output signal based on a set of logical rules. Logic gates can be classified into the following categories: - Basic Gates: - AND Gate: The AND gate produces a high output (1) only when all of its inputs are high (1). - OR Gate: The OR gate produces a high output (1) if any of its inputs is high (1). - NOT Gate (Inverter): The NOT gate produces the logical complement of its input. It takes a single input and produces the opposite value as the output. - Derived Gates: - NAND Gate: The NAND gate is a combination of an AND gate followed by a NOT gate. It produces the inverse of the AND gate's output. It outputs a low (0) only when all of its inputs are high (1). - NOR Gate: The NOR gate is a combination of an OR gate followed by a NOT gate. It produces the inverse of the OR gate's output. It outputs a high (1) only when all of its inputs are low (0). - XOR Gate (Exclusive OR): The XOR gate produces a high output (1) when the number of high inputs is odd. It outputs a low (0) when the number of high inputs is even. - XNOR Gate (Exclusive NOR): The XNOR gate produces a high output (1) when the number of high inputs is even. It outputs a low (0) when the number of high inputs is odd. - Universal Gates: NAND and NOR gates are considered universal gates because any logic function can be implemented using only NAND gates or only NOR gates. This means that with a sufficient number of NAND or NOR gates, you can create circuits that can perform any logical operation.

Implementation Details

AND Gate:Y= Symbol Pin diagram Truth Table:. OR Gate: Y = Symbol Pin Diagram Truth Table:. NOT Gate: Y = Symbol Pin Diagram Truth Table:. NAND Gate: Y = Symbol Pin Diagram Truth Table:. NOR Gate: Y = Symbol Pin Diagram Truth Table:. XOR Gate: Y = Symbol. Pin Diagram. Truth Table:. XNOR Gate: Y = Symbol. Pin Diagram. Truth Table:. Implementation Using NAND Gate NOT GATE AND GATE OR GATE. Implementation Using NOR Gate NOT GATE. AND GATE. OR GATE.

Post Lab Subjective/Objective type Questions:

- Implement the Boolean function using NAND gates and NOR gates F=A’B + AB’ - Implement using combination of gates F = ABC + AB’C + ABC’

Conclusion:

Signature of faculty in-charge with Date: