| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | 22/10/23 | Batch No: | C-1 |
| Faculty Name: |  | Roll No: | 16010122257 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 5

Title: Flip Flops

Aim and Objective of the Experiment:

To Verify truth table of JK Flip flop using IC 7476 and study conversion of JK FF to D FF and T FF

COs to be achieved:

CO3: Design synchronous and asynchronous sequential circuits.

Tools used:

Trainer kits

Theory:

Flip-flop is the common name given to two-state devices which offer basic memory for sequential logic operations. Flip-flops are heavily used for digital data storage and transfer and are commonly used in banks called "registers" for the storage of binary numerical data. JK-flip flop: has two inputs, traditionally labeled J and K. IC 7476 is a dual JK master slave flip flop with preset and clear inputs. If J and K are different then the output Q takes the value of J at the next clock edge. If J and K are both low then no change occurs. If J and K are both high at the clock edge, then the output will toggle from one state to the other. It can perform the functions of the set/reset flip-flop and has the advantage that there are no ambiguous states. D Flip Flop: tracks the input, making transitions with match those of the input D. The D stands for "data"; this flip-flop stores the value that is on the data line. It can be thought of as a basic memory cell. D flip-flop can be made from J-K flip-flop by connecting both inputs through a not gate. T Flip Flop: T or "toggle" flip-flop changes its output on each clock edge, giving an output which is half the frequency of the signal to the T input. It is useful for constructing binary counters, frequency dividers, and general binary addition devices. It can be made from a J-K flip-flop by tying both of its inputs high. Implementation Details: Procedure - Locate IC 7476 on Digital trainer kit - Apply various inputs to J & K pins by means of the output on logic output indicator. - Connect a pulsar switch to the clock input. Connect the J&K as D and T flip flop as shown in diagrams and verify the respective truth tables.

- Logic Symbol Pin Diagram of IC 7476 Truth Table of JK FF Conversion of FFs - JK to D FF Conversion Diagram Truth Table of D FF CLK; D; Q. >>0; X; Q; Q’. >>1; 0; 0; 1. >>1; 1; 1; 0. - JK to T FF Conversion Diagram Truth Table of T FF CLK; T; Q. 0; Q; Q’. 1; Q’; Q. X; Q; Q’.

Implementation Details

Procedure: - Locate the IC 7476 and place the IC on trainer kit. - Connect VCC and ground to respective pins of IC trainer kit. - Implement the circuit as shown in the circuit diagram. - Connect the inputs to the input switches provided in the trainer kit. - Connect the outputs to the switches of O/P LEDs - Apply various combinations of inputs according to the truth table and observe the condition of LEDs. - Note down the corresponding output readings for various combinations of inputs.

Post Lab Subjective/Objective type Questions:

- How does a JK flip-flop differ from an SR flip-flop in its basic operation? - What is the use of characteristic and excitation table? - How many flip flops do you require storing the data 1101?

Conclusion:

Signature of faculty in-charge with Date: